RISK ASSESSMENT (RA)                                                                                                   Pulling & Running Operations

تقييم المخاطر عمليات السحب والإنزال

Well Services Department

Kamra for Energy & Contracting L.L.C

Risk Assessment Manual

Contents

# Document Control & Revision History

# التحكم بالوثيقة وسجل التعديلات

## 1. Document Information | معلومات الوثيقة

| Item | Details |
| --- | --- |
| Document Title | Risk Assessment – Pulling & Running Operations |
| Document Number | RA-WS-001 |
| Revision | Rev.00 |
| Department | Well Services Department |
| Document Owner | Kamra for Energy & Contracting L.L.C |
| Document Type | Risk Assessment (RA) |
| Document Status | Approved |
| Effective Date | 22 / 07 / 2026 |
| Review Date | 22 / 07 / 2027 |
| Validity | 12 Months or upon any significant operational change |

## 2. Revision History | سجل التعديلات

| Rev. | Date | Description | Prepared By | Reviewed By | Approved By |
| --- | --- | --- | --- | --- | --- |
| 00 | 22 / 07 / 2026 | Initial Issue | Well Services Department | HSE Manager | General Manager |
| 01 | — | Future Revision | — | — | — |
| 02 | — | Future Revision | — | — | — |

## 3. Distribution List | قائمة التوزيع

This document shall be distributed to the following personnel and departments:

يتم توزيع هذه الوثيقة على الجهات والأقسام التالية:

- General Manager

- Operations Manager

- Well Services Supervisor

- HSE Department

- Quality Department

- Client Representative (When Required)

- Wellsite Supervisors

- Company Document Control

## 4. Document Review and Approval | مراجعة واعتماد الوثيقة

This Risk Assessment shall be reviewed at least once every twelve (12) months, or whenever any of the following occurs:

- Changes in operational procedures.

- Introduction of new equipment or technology.

- Changes in applicable legal or client requirements.

- Following an incident, near miss, or significant finding.

- Upon client request or management review.

## تتم مراجعة هذا التقييم في الحالات التالية:

- عند تعديل إجراءات التشغيل.

- عند إدخال معدات أو تقنيات جديدة.

- عند تغيير متطلبات العميل أو الأنظمة.

- بعد وقوع حادث أو شبه حادث.

- عند طلب الإدارة أو العميل.

### Document Approval

| Prepared By | Reviewed By | Approved By |
| --- | --- | --- |
| Well Services Department | HSE Manager | General Manager |

Document Classification: CONTROLLED COPY

# 1. Purpose | الغرض

## 1.1 Arabic

يهدف تقييم المخاطر (Risk Assessment) إلى تحديد جميع المخاطر المحتملة المرتبطة بعمليات Pulling & Running Operations، وتقييم مستوى الخطورة، وتطبيق إجراءات التحكم المناسبة للحد من احتمالية وقوع الحوادث أو تقليل آثارها، بما يضمن سلامة العاملين، وحماية المعدات، والمحافظة على البيئة، والامتثال لمتطلبات الشركة والعميل والمعايير الدولية.

## 1.1 English

The purpose of this Risk Assessment is to identify all hazards associated with Pulling & Running Operations, evaluate the level of risk, and implement appropriate control measures to eliminate or reduce risks to an acceptable level, ensuring personnel safety, equipment integrity, environmental protection, and compliance with company, client, and international standards.

# 2. Scope | النطاق

## Arabic 2.1

ينطبق هذا التقييم على جميع عمليات Pulling & Running Operations التي تنفذها شركة Kamra for Energy & Contracting L.L.C، ويشمل جميع العاملين والمشرفين والفنيين وموجهي الإشارات، بالإضافة إلى جميع المعدات والأدوات المستخدمة في مواقع العمل.

## 2.1 English

This Risk Assessment applies to all Pulling & Running Operations performed by Kamra for Energy & Contracting L.L.C and covers supervisors, operators, technicians, riggers, HSE personnel, and all equipment and tools used during well service operations.

# 3. Objectives | الأهداف

## Arabic

- حماية العاملين من الإصابات والأمراض المهنية.

- منع تلف المعدات والأدوات.

- حماية البيئة من التلوث.

- تقليل المخاطر التشغيلية إلى أدنى مستوى عملي ممكن.

- ضمان الالتزام بإجراءات السلامة.

- دعم الامتثال لمتطلبات العميل.

- تحسين الكفاءة التشغيلية وتقليل التوقفات غير المخططة.

## English

- Protect personnel from injury and occupational illness.

- Prevent damage to equipment and assets.

- Protect the environment from pollution.

- Reduce operational risks to As Low As Reasonably Practicable (ALARP).

- Ensure compliance with safety procedures.

- Meet client and company requirements.

- Improve operational efficiency and minimize non-productive time.

# 4. Risk Assessment Methodology | منهجية تقييم المخاطر

يتم تقييم المخاطر وفق الخطوات التالية:

- تحديد المهمة (Task).

- تحديد الخطر (Hazard).

- تحديد العواقب المحتملة (Potential Consequences).

- تحديد الأشخاص أو الممتلكات المعرضة للخطر.

- تقييم الاحتمالية (Likelihood).

- تقييم شدة العواقب (Severity).

- حساب مستوى الخطورة (Risk Rating).

- تحديد إجراءات التحكم (Control Measures).

- إعادة تقييم مستوى الخطورة بعد تطبيق إجراءات التحكم (Residual Risk).

- مراجعة واعتماد التقييم قبل بدء العمل.

## Risk Assessment Process

- Identify the task.

- Identify the hazard.

- Identify potential consequences.

- Identify exposed personnel or assets.

- Assess likelihood.

- Assess severity.

- Determine the risk rating.

- Apply control measures.

- Reassess the residual risk.

- Obtain approval before commencing work.

# 4. Risk Assessment Matrix & Evaluation Method

# مصفوفة تقييم المخاطر وآلية التقييم

# 4.1 Risk Assessment Method

## 4.1 منهجية تقييم المخاطر

### English

Risk assessment is performed by evaluating:

- Likelihood (Probability of occurrence)

- Severity (Impact of consequences)

The overall Risk Rating is determined using the following formula:

Risk Rating = Likelihood × Severity

The calculated risk level shall be used to determine the required control measures

before commencing the work.

### العربية

يتم تقييم مستوى المخاطر من خلال تحديد

- احتمالية الحدوث (Likelihood)

- شدة العواقب (Severity)

ويتم حساب مستوى الخطورة وفق المعادلة التالية:

مستوى الخطورة = احتمالية الحدوث × شدة العواقب

ويستخدم ناتج التقييم لتحديد إجراءات التحكم المناسبة قبل البدء بالعمل.

# 4.2 Likelihood Scale

## مقياس احتمالية الحدوث

| Rating | Description | الوصف |
| --- | --- | --- |
| 1 | Rare | نادر جداً |
| 2 | Unlikely | غير محتمل |
| 3 | Possible | محتمل |
| 4 | Likely | محتمل بدرجة كبيرة |
| 5 | Almost Certain | شبه مؤكد |

# 4.3 Severity Scale

## مقياس شدة العواقب

| Rating | Description | الوصف |
| --- | --- | --- |
| 1 | Insignificant | تأثير بسيط جداً |
| 2 | Minor | إصابة أو ضرر بسيط |
| 3 | Moderate | إصابة متوسطة أو تلف متوسط |
| 4 | Major | إصابة خطيرة أو تلف كبير |
| 5 | Catastrophic | وفاة أو خسائر جسيمة |

# 4.4 Risk Matrix

## مصفوفة تقييم المخاطر

| Severity ↓ / Likelihood → | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 5 Catastrophic | 5 | 10 | 15 | 20 | 25 |
| 4 Major | 4 | 8 | 12 | 16 | 20 |
| 3 Moderate | 3 | 6 | 9 | 12 | 15 |
| 2 Minor | 2 | 4 | 6 | 8 | 10 |
| 1 Insignificant | 1 | 2 | 3 | 4 | 5 |

# 4.5 Risk Classification

## تصنيف مستوى الخطورة

| Risk Score | Risk Level | الإجراء المطلوب |
| --- | --- | --- |
| 1 – 4 | Low | يمكن مباشرة العمل مع تطبيق إجراءات السلامة الأساسية. |
| 5 – 9 | Medium | يتطلب تطبيق إجراءات تحكم إضافية ومراقبة مستمرة. |
| 10 – 16 | High | يجب تنفيذ إجراءات تحكم صارمة وموافقة المشرف قبل بدء العمل. |
| 17 – 25 | Extreme | يمنع بدء العمل حتى يتم تخفيض مستوى الخطورة إلى مستوى مقبول. |

# 4.6 ALARP Principle

## مبدأ ALARP

### English

All risks shall be reduced to As Low As Reasonably Practicable (ALARP) before work commences. Where the residual risk remains High or Extreme, additional engineering, administrative, or procedural controls shall be implemented.

### العربية

يجب خفض جميع المخاطر إلى أدنى مستوى عملي يمكن تحقيقه (ALARP) قبل البدء بالعمل. وإذا بقيت الخطورة مرتفعة أو حرجة بعد تطبيق إجراءات التحكم، فيجب اتخاذ إجراءات إضافية قبل السماح بتنفيذ المهمة.

# 5. Risk Assessment Register

# سجل تقييم المخاطر

## Activity:

Pulling & Running Operations – Preparation Stage

| No. | Task / المهمة | Hazard / الخطر | Potential Consequences / العواقب المحتملة | Existing Control Measures / إجراءات التحكم الحالية | Likelihood | Severity | Risk | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Review Work Plan | Failure to review approved procedures | Incorrect operation, equipment damage | Review approved work program and supervisor verification | 3 | 3 | 9 | 3 | Supervisor |
| 2 | Conduct Toolbox Talk | Poor communication | Unsafe acts and misunderstanding | Daily Toolbox Talk before work | 3 | 3 | 9 | 3 | Supervisor / HSE |
| 3 | Risk Assessment (RA/JSA) | Hazards not identified | Injury or property damage | Complete RA/JSA before work starts | 4 | 4 | 16 | 4 | Supervisor / HSE |
| 4 | Verify PTW | Working without authorization | Serious incident or non-compliance | Approved Permit to Work where required | 3 | 4 | 12 | 4 | Supervisor |
| 5 | Inspect Equipment | Equipment defects | Equipment failure and injuries | Pre-use inspection and maintenance records | 4 | 4 | 16 | 4 | Operator |
| 6 | Verify Lifting Equipment | Lifting gear failure | Dropped objects and severe injuries | Valid inspection certificates and SWL/WLL verification | 4 | 5 | 20 | 5 | Lifting Supervisor |
| 7 | Verify PPE | Failure to use PPE | Personal injuries | Mandatory PPE inspection before work | 3 | 4 | 12 | 3 | HSE Officer |
| 8 | Barricade Work Area | Unauthorized entry | Personnel exposure to hazards | Install barriers and warning signs | 3 | 4 | 12 | 3 | Supervisor |
| 9 | Emergency Preparedness | Emergency equipment unavailable | Delayed emergency response | Verify ERP, fire extinguishers and first aid equipment | 2 | 5 | 10 | 4 | HSE Officer |
| 10 | Communication Check | Communication failure | Delayed response during operations | Test radios and communication devices before work | 3 | 3 | 9 | 3 | Supervisor |

## General Control Requirements

### متطلبات التحكم العامة

### English

- Conduct a Risk Assessment (RA) or Job Safety Analysis (JSA) before starting work.

- Ensure all personnel are trained and authorized.

- Verify the validity of inspection and calibration certificates.

- Ensure all lifting equipment complies with SWL/WLL requirements.

- Conduct Toolbox Talk before every shift.

- Wear the required Personal Protective Equipment (PPE).

- Establish exclusion zones using barriers and warning signs.

- Confirm emergency response arrangements before commencing work.

### العربية

- إجراء تقييم المخاطر (RA) أو تحليل سلامة العمل (JSA) قبل بدء العمل.

- التأكد من أن جميع العاملين مدربون ومصرح لهم.

- التحقق من صلاحية شهادات الفحص والمعايرة.

- التأكد من مطابقة معدات الرفع لمتطلبات SWL/WLL.

- عقد اجتماع السلامة (Toolbox Talk) قبل كل وردية.

- ارتداء معدات الوقاية الشخصية المناسبة.

- عزل منطقة العمل باستخدام الحواجز واللافتات التحذيرية.

- التأكد من جاهزية خطة الاستجابة للطوارئ قبل بدء العمليات.

# 6. Risk Assessment Register

# سجل تقييم المخاطر

## Activity:

Pulling & Running Operations – Equipment Preparation Stage

| No. | Task / المهمة | Hazard / الخطر | Potential Consequences / العواقب المحتملة | Existing Control Measures / إجراءات التحكم الحالية | Likelihood | Severity | Risk | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Unload Equipment | Dropped objects | Serious injury or equipment damage | Use certified lifting equipment and establish an exclusion zone | 4 | 5 | 20 | 5 | Lifting Supervisor |
| 2 | Move Equipment to Work Area | Collision with personnel or equipment | Injury and property damage | Use banksman and designated travel routes | 3 | 4 | 12 | 4 | Operator |
| 3 | Position Equipment | Unstable ground or poor positioning | Equipment overturning | Verify ground stability and level the equipment | 3 | 5 | 15 | 5 | Supervisor |
| 4 | Connect Hydraulic Hoses | Hydraulic oil leak | Injection injury, slip hazard, environmental contamination | Inspect hoses and fittings before connection | 3 | 4 | 12 | 4 | Operator |
| 5 | Connect Power Supply | Electrical hazard | Electric shock or equipment damage | Inspect cables, ensure proper grounding and isolation | 2 | 5 | 10 | 4 | Electrician |
| 6 | Inspect Pressure Lines | Burst hose or fitting failure | High-pressure release causing injury | Pressure-test and inspect all pressure components | 3 | 5 | 15 | 5 | Supervisor |
| 7 | Verify Safety Devices | Safety device failure | Increased operational risk | Test emergency stop, alarms and interlocks before operation | 2 | 5 | 10 | 3 | Operator |
| 8 | Check Tools and Accessories | Defective tools | Hand injuries or equipment failure | Remove damaged tools from service and replace immediately | 3 | 3 | 9 | 3 | Operator |
| 9 | Housekeeping | Trip and slip hazards | Minor or serious injuries | Keep work area clean and free from obstacles | 3 | 3 | 9 | 2 | All Personnel |
| 10 | Final Pre-Start Inspection | Undetected equipment defect | Equipment failure during operation | Complete and sign the pre-start inspection checklist | 3 | 4 | 12 | 3 | Supervisor |

# Additional Control Measures

## إجراءات التحكم الإضافية

### English

- Ensure all equipment has a valid inspection certificate.

- Verify Safe Working Load (SWL) and Working Load Limit (WLL) before lifting.

- Never stand beneath suspended loads.

- Isolate all energy sources before connecting or disconnecting equipment.

- Immediately clean any hydraulic oil spills.

- Ensure emergency stop devices are fully operational.

- Maintain continuous communication between the operator, supervisor, and banksman.

- Stop work immediately if any unsafe condition is identified.

### العربية

- التأكد من أن جميع المعدات تحمل شهادات فحص سارية.

- التحقق من حدود الحمل الآمن (SWL) وحدود حمل التشغيل (WLL) قبل أي عملية رفع.

- يمنع الوقوف أو المرور أسفل الأحمال المعلقة.

- عزل جميع مصادر الطاقة قبل توصيل أو فصل المعدات.

- تنظيف أي تسرب للزيت الهيدروليكي فورًا.

- اختبار زر الإيقاف الاضطراري وأنظمة الإنذار قبل التشغيل.

- الحفاظ على التواصل المستمر بين المشغل والمشرف وموجّه الإشارات.

- إيقاف العمل فورًا عند اكتشاف أي حالة غير آمنة.

## Required PPE

### معدات الوقاية الشخصية المطلوبة

- Safety Helmet

- Safety Glasses

- Cut-Resistant Gloves

- Safety Boots

- Flame Resistant Coverall (FR)

- Hearing Protection (when required)

ملاحظة: لا يجوز الانتقال إلى مرحلة التشغيل إلا بعد التأكد من اكتمال جميع متطلبات هذه المرحلة واعتماد المشرف لنتائج الفحص

# 7. Risk Assessment Register

## Activity:

Pulling & Running Operations – Start-up & Operational Readiness

# سجل تقييم المخاطر

| No. | Task / المهمة | Hazard / الخطر | Potential Consequences / العواقب المحتملة | Existing Control Measures / إجراءات التحكم الحالية | Likelihood | Severity | Risk | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Pre-Start Meeting | Incomplete communication | Operational errors and unsafe acts | Conduct pre-start briefing with all personnel | 3 | 3 | 9 | 3 | Supervisor |
| 2 | Verify Work Area | Unauthorized personnel in work zone | Personnel injury | Establish exclusion zone and warning signs | 3 | 4 | 12 | 3 | HSE Officer |
| 3 | Start Hydraulic System | Sudden pressure build-up | Hose rupture or equipment damage | Increase pressure gradually and monitor gauges | 3 | 5 | 15 | 5 | Operator |
| 4 | Functional Test | Equipment malfunction | Unexpected movement or failure | Perform function test before full operation | 3 | 4 | 12 | 4 | Operator |
| 5 | Verify Communication | Communication failure | Delayed emergency response | Test radios and maintain continuous communication | 2 | 4 | 8 | 2 | Supervisor |
| 6 | Pressure Monitoring | Overpressure | Equipment failure or personnel injury | Monitor pressure continuously and stay within operating limits | 3 | 5 | 15 | 5 | Operator |
| 7 | Emergency Stop Test | Emergency stop not functioning | Inability to stop equipment safely | Test emergency stop system before operation | 2 | 5 | 10 | 3 | Operator |
| 8 | Personnel Positioning | Standing near moving equipment | Crushing or struck-by injuries | Maintain safe distance and follow exclusion zones | 3 | 5 | 15 | 4 | All Personnel |
| 9 | Environmental Conditions | Poor visibility, high winds, rain | Reduced operational safety | Suspend work if conditions become unsafe | 3 | 4 | 12 | 4 | Supervisor |
| 10 | Authorization to Proceed | Operation started without approval | Unsafe operation | Supervisor confirms all checks are complete before start-up | 2 | 5 | 10 | 2 | Supervisor |

# Critical Safety Requirements

## متطلبات السلامة الحرجة

### English

- Conduct a final pre-start inspection before energizing the system.

- Ensure all personnel are outside the danger zone.

- Increase hydraulic pressure gradually.

- Continuously monitor pressure gauges during start-up.

- Never bypass safety devices or alarms.

- Maintain constant communication between all team members.

- Stop the operation immediately if abnormal noise, vibration, leakage, or pressure fluctuations are observed.

- Only the authorized operator may start the equipment.

### العربية

- إجراء فحص نهائي قبل تشغيل النظام.

- التأكد من خروج جميع العاملين من منطقة الخطر.

- رفع الضغط الهيدروليكي تدريجيًا.

- مراقبة مؤشرات الضغط بشكل مستمر أثناء بدء التشغيل.

- يمنع تعطيل أو تجاوز أجهزة السلامة أو الإنذار.

- الحفاظ على التواصل المستمر بين جميع أفراد الفريق.

- إيقاف العملية فورًا عند ملاحظة أي صوت غير طبيعي أو اهتزاز أو تسرب أو تغير غير طبيعي في الضغط.

- لا يُسمح بتشغيل المعدات إلا بواسطة الشخص المخول.

## Required Documentation Before Start-up

### المستندات المطلوبة قبل بدء التشغيل

- Approved Risk Assessment (RA)

- Job Safety Analysis (JSA)

- Permit to Work (PTW) – if applicable

- Toolbox Talk Record

- Equipment Inspection Checklist

- Lifting Equipment Certificates (where applicable)

- Pre-Start Inspection Checklist

Note: No operation shall commence until all safety checks, inspections, and approvals have been completed and verified by the responsible supervisor.

ملاحظة: لا يجوز البدء بأي عملية حتى يتم استكمال جميع الفحوصات وإجراءات السلامة والحصول على موافقة المشرف المسؤول

# 8. Risk Assessment Register

# سجل تقييم المخاطر

## Activity:

Pulling & Running Operations – Operational Activities

| No. | Task / المهمة | Hazard / الخطر | Potential Consequences / العواقب المحتملة | Existing Control Measures / إجراءات التحكم الحالية | Likelihood | Severity | Risk | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Pulling & Running Tubulars | Suspended load | Fatal injury or equipment damage | Certified lifting equipment, exclusion zone, qualified banksman | 4 | 5 | 20 | 5 | Lifting Supervisor |
| 2 | Handling Tubulars | Pinch points | Hand and finger injuries | Keep hands clear, use handling tools and cut-resistant gloves | 3 | 4 | 12 | 3 | Operator |
| 3 | Operating Hydraulic Equipment | High hydraulic pressure | Hose rupture, oil injection injury | Monitor pressure gauges, inspect hoses, stay within operating limits | 3 | 5 | 15 | 5 | Operator |
| 4 | Rotating Equipment | Entanglement | Serious injury | Keep loose clothing away, install guards, maintain safe distance | 3 | 5 | 15 | 4 | Operator |
| 5 | Working at Height (if applicable) | Fall from height | Serious injury or fatality | Approved fall protection system and inspected anchor points | 3 | 5 | 15 | 4 | Supervisor |
| 6 | Manual Handling | Musculoskeletal injury | Back strain and sprains | Apply safe lifting techniques and use mechanical aids where possible | 3 | 3 | 9 | 3 | All Personnel |
| 7 | Dropped Objects | Falling tools or equipment | Injury to personnel below | Secure all tools and establish exclusion zones | 4 | 5 | 20 | 5 | All Personnel |
| 8 | Hydraulic Oil Leak | Slip hazard and environmental contamination | Personnel injury and pollution | Stop operation, contain spill, clean immediately | 3 | 4 | 12 | 3 | HSE Officer |
| 9 | Unexpected Equipment Failure | Mechanical breakdown | Equipment damage and personnel injury | Stop operation immediately and isolate equipment | 3 | 5 | 15 | 4 | Supervisor |
| 10 | Poor Communication | Incorrect operation | Collision, delay or injury | Maintain continuous radio communication and standard hand signals | 3 | 4 | 12 | 3 | Supervisor |

# Operational Control Measures

## إجراءات التحكم أثناء التشغيل

### English

- Operate equipment only within the manufacturer’s operating limits.

- Maintain continuous communication between the operator, supervisor and banksman.

- Never stand under suspended loads.

- Keep hands and body away from moving and rotating parts.

- Monitor hydraulic pressure throughout the operation.

- Stop work immediately if abnormal vibration, noise, leakage or equipment malfunction is observed.

- Suspend operations during unsafe weather conditions.

- Maintain good housekeeping throughout the work area.

### العربية

- تشغيل المعدات ضمن الحدود التشغيلية المحددة من الشركة المصنعة.

- الحفاظ على التواصل المستمر بين المشغل والمشرف وموجّه الإشارات.

- يمنع الوقوف أو المرور أسفل الأحمال المعلقة.

- إبعاد اليدين والجسم عن الأجزاء المتحركة والدوارة.

- مراقبة ضغط النظام الهيدروليكي باستمرار.

- إيقاف العمل فورًا عند ملاحظة أي اهتزاز أو ضوضاء أو تسرب أو عطل غير طبيعي.

- إيقاف العمل عند سوء الأحوال الجوية إذا أثرت على السلامة.

- المحافظة على نظافة وترتيب موقع العمل طوال فترة التشغيل.

# Mandatory PPE During Operation

## معدات الوقاية الشخصية الإلزامية أثناء التشغيل

- Safety Helmet

- Safety Glasses

- Flame Resistant (FR) Coverall

- Cut-Resistant Gloves

- Steel Toe Safety Boots

- Hearing Protection (where required)

- Fall Protection Equipment (when working at height)

## Operational Stop Criteria

### معايير إيقاف العمل

يجب إيقاف العمل فورًا في الحالات التالية:

- فقدان الاتصال بين أفراد الفريق.

- تجاوز الضغط الحدود التشغيلية المسموح بها.

- حدوث تسرب هيدروليكي.

- تعطل أي جهاز من أجهزة السلامة.

- سقوط أدوات أو معدات داخل منطقة العمل.

- تغير الأحوال الجوية بما يؤثر على سلامة العمليات.

- أي حالة غير آمنة يحددها المشرف أو مسؤول السلامة.

Authority to Stop Work: Every employee has the authority and responsibility to stop the work if an unsafe condition or unsafe act is observed.

# 9. Risk Assessment Register

# سجل تقييم المخاطر

## Activity:

Pulling & Running Operations – Completion, Shutdown & Demobilization

| - No. | Task / المهمة | Hazard / الخطر | Potential Consequences / العواقب المحتملة | Existing Control Measures / إجراءات التحكم الحالية | Likelihood | Severity | Risk | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Stop Equipment | Unexpected equipment movement | Personnel injury | Follow approved shutdown procedure and isolate all energy sources | 2 | 5 | 10 | 2 | Operator |
| 2 | Release Hydraulic Pressure | Residual pressure | High-pressure fluid release causing injury | Depressurize the system before disconnecting hoses | 3 | 5 | 15 | 3 | Operator |
| 3 | Disconnect Hydraulic Hoses | Hydraulic oil spill | Slip hazard and environmental contamination | Use drip trays and cap hose ends immediately | 3 | 4 | 12 | 3 | Operator |
| 4 | Remove Equipment | Dropped objects | Serious injury or equipment damage | Use certified lifting equipment and qualified lifting personnel | 3 | 5 | 15 | 4 | Lifting Supervisor |
| 5 | Load Equipment onto Transport | Load shift during transportation | Vehicle accident or equipment damage | Secure loads using approved restraints and verify load stability | 3 | 5 | 15 | 4 | Driver / Supervisor |
| 6 | Housekeeping | Remaining waste or spilled fluids | Slip, trip, environmental pollution | Remove waste, clean spills and dispose of waste properly | 3 | 3 | 9 | 2 | All Personnel |
| 7 | Final Site Inspection | Unidentified hazards left on site | Injury to personnel or client | Conduct a final inspection with the client representative if required | 2 | 4 | 8 | 2 | Supervisor |
| 8 | Waste Management | Improper disposal of waste | Environmental impact and regulatory non-compliance | Segregate and dispose of waste according to company procedures | 2 | 4 | 8 | 2 | HSE Officer |
| 9 | Documentation | Incomplete records | Loss of operational traceability | Complete all reports and inspection records before leaving site | 2 | 3 | 6 | 2 | Supervisor |
| 10 | Close Permit to Work | Permit left open | Administrative non-compliance | Verify work completion and formally close the PTW | 2 | 4 | 8 | 2 | Supervisor |

# Completion Control Measures

## إجراءات التحكم عند إنهاء العمل

### English

- Shut down equipment in accordance with the approved operating procedure.

- Isolate all energy sources before disconnecting equipment.

- Ensure hydraulic pressure has been completely released.

- Remove all tools, equipment and waste materials from the work area.

- Restore the worksite to a safe and clean condition.

- Complete all required reports and obtain client sign-off where applicable.

- Verify that all permits have been closed before leaving the site.

### العربية

- إيقاف المعدات وفقًا لإجراءات التشغيل المعتمدة.

- عزل جميع مصادر الطاقة قبل فصل المعدات.

- التأكد من تفريغ الضغط الهيدروليكي بالكامل.

- إزالة جميع الأدوات والمعدات والمخلفات من منطقة العمل.

- إعادة الموقع إلى حالة آمنة ونظيفة.

- استكمال جميع التقارير والسجلات المطلوبة والحصول على اعتماد العميل عند الحاجة.

- التأكد من إغلاق جميع تصاريح العمل قبل مغادرة الموقع.

# Post-Job Verification

## التحقق بعد انتهاء العمل

يجب التأكد من:

- ✔ جميع المعدات أُزيلت من الموقع.

- ✔ لا يوجد تسرب للزيوت أو السوائل.

- ✔ تم تنظيف الموقع بالكامل.

- ✔ تم توثيق أي ملاحظات أو حالات عدم مطابقة.

- ✔ تم إغلاق جميع تصاريح العمل.

- ✔ تم تسليم الموقع بحالة آمنة.

## Final Statement

### البيان الختامي

This Risk Assessment shall remain valid only while the work scope, equipment, personnel, and operating conditions remain unchanged. Any significant change shall require the Risk Assessment to be reviewed and re-approved before work resumes.

العربية:

يبقى تقييم المخاطر هذا ساريًا فقط ما دامت طبيعة العمل والمعدات والأفراد وظروف التشغيل دون تغيير. وفي حال حدوث أي تغيير جوهري، يجب مراجعة تقييم المخاطر واعتماده مرة أخرى قبل استئناف العمل.

# 10. Emergency Response & Environmental Controls

# الاستجابة للطوارئ والضوابط البيئية

# 10.1 Emergency Response Plan

## خطة الاستجابة للطوارئ

### Objective | الهدف

English

To ensure that all personnel respond promptly and effectively to emergencies, minimizing injuries, damage to equipment, and environmental impact.

العربية

ضمان استجابة جميع العاملين بسرعة وفعالية في حالات الطوارئ، للحد من الإصابات والأضرار بالمعدات وحماية البيئة.

# 10.2 Emergency Contact Responsibilities

## مسؤوليات الاستجابة للطوارئ

| Position | Responsibility |
| --- | --- |
| Site Supervisor | Stop work, secure the area, notify management. |
| HSE Officer | Coordinate emergency response and first aid. |
| First Aider | Provide immediate medical assistance. |
| Operator | Shut down equipment safely. |
| All Personnel | Follow emergency procedures and assemble at the designated muster point. |

# 10.3 Emergency Situations

## حالات الطوارئ المحتملة

| Emergency | Immediate Action |
| --- | --- |
| Fire | Stop work, raise the alarm, isolate equipment if safe, use a suitable fire extinguisher only if trained, evacuate if required. |
| Personnel Injury | Stop work immediately, provide first aid, notify the supervisor, and arrange medical assistance. |
| Hydraulic Oil Leak | Stop equipment, isolate the source if safe, contain the spill using spill kits, and report the incident. |
| Dropped Object | Secure the area, prevent entry, inspect for injuries or damage, and investigate before resuming work. |
| Equipment Failure | Stop the operation, isolate the equipment, report the fault, and do not restart until approved. |
| Severe Weather | Suspend operations and move personnel to a safe location until conditions improve. |

# 10.4 Environmental Protection

## حماية البيئة

### English

- Prevent oil and chemical spills.

- Use drip trays during hose disconnection.

- Dispose of waste according to company procedures.

- Segregate hazardous and non-hazardous waste.

- Report all environmental incidents immediately.

- Keep spill response equipment available at the worksite.

### العربية

- منع انسكاب الزيوت والمواد الكيميائية.

- استخدام صواني تجميع السوائل عند فصل الخراطيم.

- التخلص من النفايات وفق إجراءات الشركة.

- فصل النفايات الخطرة عن غير الخطرة.

- الإبلاغ الفوري عن أي حادث بيئي.

- التأكد من توفر معدات مكافحة الانسكابات في موقع العمل.

# 10.5 Emergency Equipment Checklist

## قائمة معدات الطوارئ

| Equipment | Status (✓ / ✗) |
| --- | --- |
| Fire Extinguisher |  |
| First Aid Kit |  |
| Spill Kit |  |
| Emergency Shower / Eyewash (if applicable) |  |
| Emergency Stop System |  |
| Two-Way Radio |  |
| Warning Signs & Barricades |  |
| Emergency Contact List |  |

# 10.6 Incident Reporting

## الإبلاغ عن الحوادث

### English

All incidents, near misses, environmental releases, equipment failures, and unsafe conditions shall be reported immediately to the Site Supervisor and the HSE Department. An Incident Report shall be completed before the end of the shift.

### العربية

يجب الإبلاغ فورًا عن جميع الحوادث، وشبه الحوادث، والانسكابات البيئية، وأعطال المعدات، والحالات غير الآمنة إلى مشرف الموقع وقسم الصحة والسلامة والبيئة (HSE). كما يجب استكمال تقرير الحادث (Incident Report) قبل نهاية الوردية.

# 10.7 Stop Work Authority

## صلاحية إيقاف العمل

Every employee has both the authority and responsibility to stop any activity that presents an unacceptable risk to personnel, equipment, or the environment. Work may only resume after the hazard has been eliminated or controlled and authorization has been given by the responsible supervisor.

لكل موظف الحق والمسؤولية في إيقاف أي عمل يشكل خطرًا غير مقبول على الأشخاص أو المعدات أو البيئة. ولا يجوز استئناف العمل إلا بعد إزالة الخطر أو السيطرة عليه والحصول على موافقة المشرف المسؤول.

# 11. Monitoring, Review & Approval

# المتابعة والمراجعة والاعتماد

# 11.1 Monitoring and Performance Evaluation

## متابعة وتقييم الأداء

### English

The effectiveness of this Risk Assessment shall be continuously monitored throughout the operation to ensure that all identified hazards remain adequately controlled.

Monitoring shall include:

- Regular workplace inspections.

- Supervisor observations.

- HSE inspections and audits.

- Verification of control measures.

- Review of incidents and near misses.

- Compliance with company procedures and client requirements.

### العربية

تتم متابعة فعالية تقييم المخاطر بشكل مستمر أثناء تنفيذ العمليات للتأكد من أن جميع المخاطر المحددة لا تزال

تحت  السيطرة

وتشمل المتابعة ما يلي

- إجراء جولات تفتيش دورية في موقع العمل.

- متابعة المشرف لسير العمل.

- تنفيذ عمليات التفتيش والتدقيق بواسطة قسم HSE.

- التحقق من فعالية إجراءات التحكم.

- مراجعة الحوادث وشبه الحوادث.

- التأكد من الالتزام بإجراءات الشركة ومتطلبات العميل.

# 11.2 Review of Risk Assessment

## مراجعة تقييم المخاطر

This Risk Assessment shall be reviewed and updated whenever one or more of the following occurs:

- Introduction of new equipment.

- Changes in work procedures.

- Change of work location.

- Following an incident or near miss.

- Client request.

- New legal or regulatory requirements.

- Annual document review.

### تتم مراجعة تقييم المخاطر في الحالات التالية

- إدخال معدات جديدة.

- تعديل إجراءات العمل.

- تغيير موقع العمل.

- بعد وقوع حادث أو شبه حادث.

- بناءً على طلب العميل.

- عند صدور متطلبات أو تشريعات جديدة.

- عند المراجعة السنوية للوثيقة.

# 11.3 Document Approval

## اعتماد الوثيقة

| Position | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| HSE Manager |  |  |  |
| Operations Manager |  |  |  |
| General Manager |  |  |  |

# 11.4 Document Distribution

## توزيع الوثيقة

This document shall be issued as a Controlled Copy and distributed to:

- General Manager

- Operations Manager

- HSE Department

- Well Services Department

- Supervisors

- Client Representative (when required)

- Document Control

# 11.5 Document Retention

## حفظ الوثيقة

The approved Risk Assessment shall be retained in accordance with the Company’s Document Control Procedure and shall remain available throughout the duration of the project.

### العربية

يتم حفظ النسخة المعتمدة من تقييم المخاطر وفقًا لإجراءات ضبط الوثائق الخاصة بالشركة، وتبقى متاحة طوال مدة المشروع، مع الاحتفاظ بالإصدارات السابقة لأغراض التتبع والمراجعة.

# Final Declaration

## الإقرار النهائي

This Risk Assessment has been prepared to identify, evaluate, and control the hazards associated with Pulling & Running Operations. All personnel involved in these activities shall read, understand, and comply with the requirements of this document before commencing work.

### العربية

### تم إعداد تقييم المخاطر هذا لتحديد وتحليل والسيطرة على المخاطر المرتبطة بعمليات   Pulling & Running Operations

### . ويجب على جميع العاملين المشاركين في هذه العمليات قراءة هذه الوثيقة وفهمها والالتزام بجميع متطلباتها قبل البدء بالعمل.

Document Status: APPROVED

Classification: CONTROLLED DOCUMENT

Kamra for Energy & Contracting L.L.C

# 12. Appendices & References

# الملاحق والمراجع

# Appendix A – Abbreviations

## الملحق (أ): الاختصارات

| Abbreviation | Meaning |
| --- | --- |
| RA | Risk Assessment |
| JSA | Job Safety Analysis |
| HSE | Health, Safety & Environment |
| PPE | Personal Protective Equipment |
| PTW | Permit to Work |
| SOP | Standard Operating Procedure |
| SWL | Safe Working Load |
| WLL | Working Load Limit |
| ERP | Emergency Response Plan |
| ALARP | As Low As Reasonably Practicable |

# Appendix B – Related Documents

## الملحق (ب): الوثائق ذات الصلة

The following documents shall be available and used together with this Risk Assessment:

- Standard Operating Procedure (SOP)

- Job Safety Analysis (JSA)

- Permit to Work (PTW)

- Toolbox Talk Record

- Daily Equipment Inspection Checklist

- Lifting Equipment Inspection Certificates

- Incident Report Form

- Near Miss Report Form

- Emergency Response Plan (ERP)

- Company HSE Manual

# Appendix C – References

## الملحق (ج): المراجع

This Risk Assessment has been prepared with reference to:

- Company HSE Management System.

- Company Standard Operating Procedures.

- Applicable Client HSE Requirements.

- ISO 45001:2018 – Occupational Health and Safety Management Systems.

- ISO 9001:2015 – Quality Management Systems.

- Manufacturer’s Equipment Manuals.

- Applicable National Regulations.

# Appendix D – Revision Record

## الملحق (د): سجل المراجعات

| Revision | Date | Description | Approved By |
| --- | --- | --- | --- |
| Rev.00 | 22/07/2026 | Initial Issue | General Manager |

# Document Declaration

## الإقرار النهائي

This document forms an integral part of the Company’s Health, Safety, Environment, and Operational Management System. All personnel involved in Pulling & Running Operations shall comply with the requirements contained herein.

Any deviation from this Risk Assessment shall require formal review and written approval before work proceeds.

### العربية

تُعد هذه الوثيقة جزءًا أساسيًا من نظام إدارة الصحة والسلامة والبيئة والتشغيل في الشركة، ويجب على جميع العاملين المشاركين Pulling & Running Operations  . الالتزام بجميع متطلباتها.

ولا يجوز إجراء أي تعديل أو استثناء من هذه الوثيقة إلا بعد مراجعة واعتماد رسمي من الإدارة المختصة.

# - Document Approval

## اعتماد الوثيقة

| Prepared By | Reviewed By | Approved By |
| --- | --- | --- |
| HSE Manager | Operations Manager | General Manager |

# 13.5 Revision Record

## سجل المراجعات

| Revision | Date | Description | Approved By |
| --- | --- | --- | --- |
| Rev.00 | __________ | Initial Issue | General Manager |

CONFIDENTIALITY STATEMENT

This document is the property of Kamra for Energy &  Contracting L.L.C. It contains confidential and proprietary information intended solely for company use No part of this                     document may be reproduced, copied   distributed, or disclosed without prior written approval  from the Company

إشعار السرية

هذه الوثيقة ملك لشركة وتحتوي على معلومات سرية ومملوكة للشركة ومخصصة للاستخدام  الداخلي فقط. ولا يجوز نسخها أو توزيعها أو الإفصاح عن أي جزء  منها دون موافقة خطية مسبقة من الشركة

Email: info@kamra-energy.com      https://www.kamra-energy.com/

## END OF DOCUMENT

HSE Management Manual  Kamra for Energy & Contracting L.L.C Document No.: RA-WS-001 Revision: Rev.00  Document Status: APPROVED CONTROLLED DOCUMENT